Operator Name: Date:

Demonstrate the following procedures whilst disconnected from the power Check blade integrity, noting specifics of TCT blade. Show where bevel setting is and operate it; stopping at detents (if applicable) and arbitrary angles.. Show where mitre setting is and operate it; stopping at detents and arbitrary angles.. Show where sliding fence is and explain how you’d determine if it’s necessary. Explain which way the teeth should face. Demonstrate the following procedures whilst connected to power Cut straight cut. Cut compound cut, moving guide if necessary. Understand the following things How to report faults (users should not fix problems themselves). Meaning of no lone working rule. Blade must be running at full speed before the material is cut. How to correctly secure work. Which PPE is required or recommended (goggles required, breathing and hearing protection recommended; gloves not to be worn). Hands should be 15cm from the blade during operation. Clamps are required when work holding is required at a distance less than 15cm. Need to disconnect power whilst inspecting the blade. Machine is used for wood only or some plastics. Users must not enter yellow box areas when another user is using a power tool in that area. Users must inform other hackspace users if they are cutting anything with higher than average respiratory risks. Dust extraction must be used where available. Arms/hands must not be placed below blade (no crossing arms for cut etc). User is responsible for cleaning up.

When competence has been achieved, tick the appropriate requirement.